实验十三、用户管理实验

1  实验目的

掌握智能锁中用户管理方法。

2  实验内容

1、打开“应用程序框架”工程。

3、编写代码，构建用户结构体。

4、编写代码，实现用户的增删改功能。

5、编写代码，在main函数中调用用户管理接口，用串口方式打印用户列表。

6、编译工程，验证用户管理功能是否正确运行。

3  实验原理

基于stm8内部eeprom，实现数据存取，从而实现用户管理。

（1）用户类设计

首先设计密码类，其关键的几个成员分别为：

- type：密码类型，如键盘、卡号、指纹等；

- length：当密码类型为字符型（如数字、卡号）时，该成员表示密码字符的长度；

- text：当密码类型为字符型（如数字、卡号）时，该成员表示密码的字符。

然后设计用户类，其关键的几个成员分别为：

- id：用户编号；

- type：用户类型，如管理员、普通用户；

- status：表示该用户是否启用；

- passWord：用户密码。

C语言中没有类的概念，所以PassWord类和User类在C语言中的实现为结构体，以下代码为具体实现。

#define PASSWORD_LENGTH 7

typedef struct{

unsigned char type;//密码类型

unsigned char length;//密码长度

unsigned char text[PASSWORD_LENGTH];//密码

}PassWordT;

typedef struct{

unsigned char id;//用户编号

unsigned char type;//用户类型

unsigned char status;//用户生效标志

PassWordT passWord;//用户密码

}UserT;

在下图中，右边是eeprom存储空间分配，其中用户列表占用地址0x1000~0x1078共120字节，用户列表后面是系统参数（如是否打开语音），系统参数后面是保留空间。本实验主要讨论用户列表的管理，如下图左边是用户管理功能框图，使用一个userList指向eeprom的用户列表起始地址，然后就可以使用eeprom的方法操作用户列表。

（2）用户管理功能设计

用户管理功能包括新增用户、修改用户密码和删除用户，由于智能锁中用户数量不多，所以不需要查询用户功能，这样还能减轻程序复杂度。

1）新增用户

首先要查询一下用户列表，是否有可用的用户id，如果有可用id，则继续执行，否则结束本程序；如果有可用用户id，则需要先初始化该用户信息，然后修改用户密码，同时根据用户输入的密码修改密码类型、密码长度等变量，最后保存该用户到eeprom中。流程图如下图所示。

2）修改用户密码

修改用户密码时针对的是已存在的用户，所以只需要根据用户id，找到该用户，修改密码后重新保存一次就可以了。流程图如下图所示。

3）删除用户

删除用户时针对的也是已存在用户，只需要根据用户id找到该用户，把用户的状态改为禁用并保存，就实现了删除用户。流程图如下图所示。

4  实验步骤

- 使用Clion切换代码到“应用程序框架”版本。

- 启动IAR集成开发环境，选择【File】->【Open Workspace】，打开该工程，如下图：

- 在User/menu目录下创建userManager.c和userManager.h，如下图所示。

- 在iar工程中的menu目录下添加userManger.c和userManager.h，如下图所示：

- 在userManager.c中加入以下代码，实现用户的增删改功能。

#include "stdio.h"

#include "stm8l15x.h"

#include "userManager.h"

#include "string.h"

#include "EventManager.h"

#include "Board.h"

#define EEPROM_STARTADDR 0x1000

static UserT *userList;

void saveToEEPROM(unsigned char *addr, unsigned char *buffer, unsigned int length) {

unsigned int i;

uint8_t *Ptr_SegAddr; //Segment pointer

Ptr_SegAddr = (uint8_t *) (addr);  //Initialize  pointer

FLASH_Unlock(FLASH_MemType_Data);

while (!FLASH_IAPSR_DUL);//如果DUL没有置位,说明解锁不成功,最好加入超时判断

asm("sim");//关闭中断

for (i = 0; i < length; i++) {

*(Ptr_SegAddr + i) = buffer[i]; //指定地址，写1 字节

}

while (!FLASH_IAPSR_EOP);//等待写操作完成,最好加入超时判断

asm("rim");//打开中断

FLASH_Lock(FLASH_MemType_Data);

}

void saveUser(UserT *user) {

saveToEEPROM((unsigned char *) (&userList[user->id]), (unsigned char *) user, sizeof(UserT));

}

//返回密码对应的用户id

unsigned char checkPassword(unsigned char id, unsigned char passwordType, unsigned char *pPassword, unsigned char length) {

unsigned char i, userCount = 0;

for (i = 0; i < MAX_USER_COUNT; i++) {

if (userList[i].status != STATUS_DELETED)userCount++;

if (userList[i].status != STATUS_ENABLE)continue;

if (userList[i].passWord.type != passwordType)continue;

if(passwordType == TYPE_FINGER_PRINT || passwordType == TYPE_FINGER_VIEN){

if(id != i)continue;

} else if(passwordType == TYPE_KEY || passwordType == TYPE_CARD || passwordType == TYPE_NB

|| passwordType == TYPE_WIFI || passwordType == TYPE_BLE){

if(userList[i].passWord.length != length)continue;

if (memcmp(userList[i].passWord.text, pPassword, length)) continue;

}

return i;

}

if (userCount == 0)return NO_USERS;

return CHECK_PASSWORD_FAIL;

}

unsigned char checkIsAdminPassword(unsigned char id, unsigned char passwordType, unsigned char *pPassword, unsigned char length) {

unsigned char i;

for (i = 0; i < MAX_USER_COUNT; i++) {

if (userList[i].type != USER_ADMIN) continue;

if (userList[i].status != ENABLE)continue;

if (userList[i].passWord.type != passwordType)continue;

if(passwordType == TYPE_FINGER_PRINT || passwordType == TYPE_FINGER_VIEN){

if(id != i)continue;

} else if(passwordType == TYPE_KEY || passwordType == TYPE_CARD ||

passwordType == TYPE_NB || passwordType == TYPE_NB || passwordType == TYPE_WIFI || passwordType == TYPE_BLE){

if(userList[i].passWord.length != length)continue;

if (memcmp(userList[i].passWord.text, pPassword, length))continue;

}

return i;

}

return CHECK_PASSWORD_FAIL;

}

unsigned char getAvailableUserId(void) {

unsigned char i;

for (i = 0; i < MAX_USER_COUNT; i++) {

if (userList[i].status == STATUS_DELETED) {

return i;

}

}

return 0xff;

}

unsigned char haveAvailableAdmin(void) {

unsigned char i;

for (i = 0; i < MAX_USER_COUNT; i++) {

if (userList[i].type == USER_ADMIN && userList[i].status == ENABLE) {

return 1;

}

}

return 0;

}

void resetUserList(void) {

unsigned char i;

UserT user = {0};

for (i = 0; i < MAX_USER_COUNT; i++) {

saveToEEPROM((unsigned char *) (&userList[i]), (unsigned char *) &user, sizeof(UserT));

}

}

void initAddress(void) {

userList = (UserT *) EEPROM_STARTADDR;

}

unsigned char getUserCount(void) {

unsigned char i;

unsigned char count = 0;

for (i = 0; i < MAX_USER_COUNT; i++) {

if (userList[i].status != STATUS_DELETED) {

count++;

}

}

return count;

}

void printUsers(void) {

unsigned char total = getUserCount();

unsigned char i;

printf("total:%d\n", total);

printf("-------------\n");

for (i = 0; i < total; i++) {

if (userList[i].status == STATUS_DELETED) continue;

printf("id:%d\n", userList[i].id);

printf("type:");

if(userList[i].type == USER_ADMIN)printf("admin\n");

else printf("general\n");

printf("passWord type:");

if(userList[i].passWord.type == TYPE_KEY)printf("key\n");

if(userList[i].passWord.type == TYPE_CARD)printf("card\n");

if(userList[i].passWord.type == TYPE_FINGER_PRINT)printf("finger print\n");

if(userList[i].passWord.type == TYPE_FINGER_VIEN)printf("finger vein\n");

if(userList[i].passWord.type == TYPE_NB)printf("NB\n");

if(userList[i].passWord.type == TYPE_WIFI)printf("WIFI\n");

if(userList[i].passWord.type == TYPE_BLE)printf("BLE\n");

printf("user.passWord.text:%s\n", userList[i].passWord.text);

printf("-------------\n");

}

}

void loadFactoryConfig(void) {

resetUserList();

}

void initUserManager(void) {

initAddress();

printUsers();

}

void initNewUser(UserT *pUser) {

pUser->id = 0;

pUser->type = USER_GENERAL;

pUser->status = ENABLE;

pUser->passWord.type = TYPE_KEY;

pUser->passWord.length = 0;

memset(pUser->passWord.text, 0, PASSWORD_LENGTH);

}

void getUsersIndexList(unsigned char *userIndexList, unsigned char *count) {

unsigned char i;

*count = 0;

for (i = 0; i < MAX_USER_COUNT; i++) {

if (userList[i].status == STATUS_DELETED) continue;

userIndexList[*count] = i;

(*count)++;

}

}

unsigned char deleteUserById(unsigned char id) {

UserT userToDelete;

memcpy(&userToDelete, &userList[id], sizeof(UserT));

userToDelete.status = STATUS_DELETED;

saveUser(&userToDelete);

return 0;

}

UserT *getUserById(unsigned char id) {

return &userList[id];

}

- 在userManager.h中定义声明函数API。

#ifndef USER_MANAGER_H_

#define USER_MANAGER_H_

typedef enum{

TYPE_KEY = 0,

TYPE_CARD,

TYPE_FINGER_PRINT,

TYPE_FINGER_VIEN,

TYPE_NB,

TYPE_WIFI,

TYPE_BLE

}PasswordType;

#define PASSWORD_LENGTH 7

typedef enum {

USER_GENERAL = 0,

USER_ADMIN

}UserType;

#define MAX_USER_COUNT 10

typedef enum{

STATUS_DELETED = 0,

STATUS_ENABLE,

STATUS_DISABLE,

} UserStatus;

#define NO_USERS 0xfe

#define CHECK_PASSWORD_FAIL 0xff

typedef struct{

unsigned char type;//密码类型

unsigned char length;//密码长度

unsigned char text[PASSWORD_LENGTH];//密码

}PassWordT;

typedef struct{

unsigned char id;  //用户编号

unsigned char type;    //用户类型

unsigned char status;  //用户生效标志

PassWordT passWord;    //用户密码

}UserT;

void initUserManager(void);

unsigned char haveAvailableAdmin(void);

unsigned char checkPassword(unsigned char id, unsigned char passwordType, unsigned char *pPassword, unsigned char length);

unsigned char checkIsAdminPassword(unsigned char id, unsigned char passwordType, unsigned char *pPassword, unsigned char length);

void saveUser(UserT *user);

void loadFactoryConfig(void);

void initNewUser(UserT *pUser);

void getUsersIndexList(unsigned char *userIndexList, unsigned char *count);

unsigned char getAvailableUserId(void);

void printUsers(void);

unsigned char deleteUserById(unsigned char id);

UserT *getUserById(unsigned char id);

#endif

- 修改main.c，实现对对用户管理功能的验证。

#include <stdio.h>

#include "string.h"

#include "stm8l15x.h"

#include "stm8_eval.h"

#include "board.h"

#include "led.h"

#include "door.h"

#include "sound.h"

#include "oled.h"

#include "tsm12.h"

#include "card.h"

#include "fingerprint.h"

#include "fingervein.h"

#include "sim7020.h"

#include "esp32.h"

#include "mainMenu.h"

#include "userManager.h"

static void newUserTest(void){

UserT newUser;

printf("-------------------newUserTest-------------------\n");

/*请在此创建并保存一个普通用户，密码类型为TYPE_KEY ，密码为“123456”*/

/*请在此创建并保存一个管理员用户，密码类型为TYPE_KEY ，密码为“654321”*/

/*注意：需要设置newUser.status 为STATUS_ENABLE*/

/*请在此打印用户列表*/

}

static void checkPasswordTest(void){

printf("-------------------checkPasswordTest-------------------\n");

/*请在此判断系统中是否有管理员用户*/

/*请在此检查系统中是否有类型为TYPE_KEY并且密码为“123456”的用户*/

/*请在此检查系统中是否有类型为TYPE_KEY并且密码为“12345”的用户*/

/*请在此检查系统中是否有类型为TYPE_KEY并且密码为“654321”的管理员用户*/

/*请在此检查系统中是否有类型为TYPE_KEY并且密码为“123456”的管理员用户*/

}

static void editUserTest(void){

printf("-------------------editUserTest-------------------\n");

UserT userToEdit;

/*请在此把id为1的用户密码类型改为TYPE_CARD ，并把密码改为“12abcd”*/

/*请在此打印用户列表*/

}

static void deleteUserTest(void){

printf("-------------------deleteUserTest-------------------\n");

/*请在此删除id为1的用户*/

/*请在此打印用户列表*/

}

void main(void) {

CLK_SYSCLKDivConfig(CLK_SYSCLKDiv_1);

boardInit();

initUserManager();

newUserTest();

checkPasswordTest();

editUserTest();

deleteUserTest();

loadFactoryConfig();

while (1) {

}

}

- 编译并烧写可执行程序到实验平台。

5  实验结果

打开串口调试助手并连接实验平台，复位一次实验平台，验证用户管理功能是否成功。