实验六、读卡模块驱动实验

1  实验目的

设计读卡模块驱动，实现读取IC卡功能。

2  实验内容

1、打开“触摸模块驱动”工程。

2、添加card模块到工程中。

3、编写代码，在board中实现rc523芯片初始化。

4、编写代码，实现从rc523芯片读取卡号，并把卡号显示在屏幕上。

5、编译工程，验证读卡模块驱动是否完成。

3  实验原理

MFRC523是高度集成的非接触式(13.56MHz)读写卡芯片，内部发送器可驱动读写器天线与ISO14443A、MIFARE卡和应答机的通信，无需其它电路。接收器提供一个功能强大和高效的解调和译码电路，用来处理兼容ISO14443A/MIFARE卡和应答机的信号。数字电路部分处理完整的ISO14443A帧和错误检测（奇偶&CRC）。MFRC523引脚封装如下图所示：

读卡模块电路如下图所示。更多相关资料请参考《6.MFRC523.pdf》。

读卡过程大致如下：MCU通过对读卡器芯片内寄存器的读写来控制读卡器芯片，读卡器芯片收到MCU发来的命令后，按照非接触式射频卡协议格式，通过天线及其匹配电路向附近发出一组固定频率的调制信号(13.56MHz)进行寻卡，若此范围内有卡片存在，卡片内部的LC谐振电路(谐振频率与读卡器发送的电磁波频率相同)在电磁波的激励下，产生共振，在卡片内部电压泵的作用下不断为其另一端的电容充电，获得能量，当该电容电压达到2V时，即可作为电源为卡片的其他电路提供工作电压。当有卡片处在读卡器的有效工作范围内时，MCU向卡片发出寻卡命令，卡片将回复卡片类型，建立卡片与读卡器的第一步联系，若同时有多张卡片在天线的工作范围内，读卡器通过启动防冲撞机制，根据卡片序列号来选定一张卡片，被选中的卡片再与读卡器进行密码校验,确保读卡器对卡片有操作权限以及卡片的合法性，而未被选中的则仍然处在闲置状态，等待下一次寻卡命令。密码验证通过之后，就可以对卡片进行读写等应用操作。

4  实验步骤

- 使用Clion切换代码到“触摸模块驱动”版本。

- 启动IAR集成开发环境，选择【File】->【Open Workspace】，打开该工程，如下图：

- 在driver目录下添加card.c和card.h，如下图所示：

- 右键【driver】->【Add】->【Add Files..】，添加card.c和card.h到工程中，如下图：

- 在card.c中加入读卡代码：

#include "stdio.h"

#include "string.h"

#include "stm8l15x.h"

#include "card.h"

#include "timer4.h"

#define RC523_PORT    GPIOF

#define SCK           GPIO_Pin_6

#define MISO          GPIO_Pin_4

#define MOSI          GPIO_Pin_5

#define NSS           GPIO_Pin_7

#define CLR_NSS GPIO_ResetBits(RC523_PORT,NSS)

#define SET_NSS GPIO_SetBits(RC523_PORT,NSS)

#define CLR_SCK GPIO_ResetBits(RC523_PORT,SCK)

#define SET_SCK GPIO_SetBits(RC523_PORT,SCK)

#define CLR_MOSI GPIO_ResetBits(RC523_PORT,MOSI)

#define SET_MOSI GPIO_SetBits(RC523_PORT,MOSI)

#define GET_MISO GPIO_ReadInputDataBit(RC523_PORT,MISO)

#define MAXRLEN 18

void rcDelay(int dly) {

int i;

for (i = 0; i < dly; i++);

}

void WriteRawRC(u8 Address, u8 value) {

u8 i, ucAddr;

CLR_SCK;

CLR_NSS;

rcDelay(5);

ucAddr = ((Address << 1) & 0x7E);

for (i = 8; i > 0; i--) {

if ((ucAddr & 0x80) == 0x80) { SET_MOSI; }

else { CLR_MOSI; }

SET_SCK;

rcDelay(5);

ucAddr <<= 1;

CLR_SCK;

rcDelay(5);

}

for (i = 8; i > 0; i--) {

if ((value & 0x80) == 0x80) {SET_MOSI;}

else {CLR_MOSI;}

SET_SCK;

rcDelay(5);

value <<= 1;

CLR_SCK;

rcDelay(5);

}

SET_NSS;

SET_SCK;

}

u8 ReadRawRC(u8 Address) {

u8 i, ucAddr;

u8 ucResult = 0;

CLR_SCK;

CLR_NSS;

rcDelay(5);

ucAddr = ((Address << 1) & 0x7E) | 0x80;

for (i = 8; i > 0; i--) {

if ((ucAddr & 0x80) == 0x80) {SET_MOSI;}

else {CLR_MOSI;}

SET_SCK;

rcDelay(5);

ucAddr <<= 1;

CLR_SCK;

rcDelay(5);

}

for (i = 8; i > 0; i--) {

SET_SCK;

rcDelay(5);

ucResult <<= 1;

if (GET_MISO) {ucResult |= 1;}

CLR_SCK;

rcDelay(5);

}

SET_NSS;

SET_SCK;

return ucResult;

}

void PcdReset(void) {

WriteRawRC(ModeReg, 0x3D);

WriteRawRC(TReloadRegL, 30);

WriteRawRC(TReloadRegH, 0);

WriteRawRC(TModeReg, 0x8D);

WriteRawRC(TPrescalerReg, 0x40);

WriteRawRC(TxAutoReg, 0x40);

}

void SetBitMask(unsigned char reg, unsigned char mask) {

char tmp = 0x0;

tmp = ReadRawRC(reg);

WriteRawRC(reg, tmp | mask);

}

void ClearBitMask(unsigned char reg, unsigned char mask) {

char tmp = 0x0;

tmp = ReadRawRC(reg);

WriteRawRC(reg, tmp & ~mask);

}

void PcdAntennaOn(void) {

unsigned char i;

i = ReadRawRC(TxControlReg);

if (!(i & 0x03)) {

SetBitMask(TxControlReg, 0x03);

}

}

void PcdAntennaOff() {

ClearBitMask(TxControlReg, 0x03);

}

void CalulateCRC(unsigned char *pIndata, unsigned char len, unsigned char *pOutData) {

unsigned char i, n;

ClearBitMask(DivIrqReg, 0x04);

WriteRawRC(CommandReg, PCD_IDLE);

SetBitMask(FIFOLevelReg, 0x80);

for (i = 0; i < len; i++) { WriteRawRC(FIFODataReg, *(pIndata + i)); }

WriteRawRC(CommandReg, PCD_CALCCRC);

i = 0xFF;

do {

n = ReadRawRC(DivIrqReg);

i--;

} while ((i != 0) && !(n & 0x04));

pOutData[0] = ReadRawRC(CRCResultRegL);

pOutData[1] = ReadRawRC(CRCResultRegM);

}

char PcdComMF522(unsigned char Command,

unsigned char *pInData,

unsigned char InLenByte,

unsigned char *pOutData,

unsigned int *pOutLenBit) {

int status = MI_ERR;

unsigned char irqEn = 0x00;

unsigned char waitFor = 0x00;

unsigned char lastBits;

unsigned char n;

unsigned int i;

switch (Command) {

case PCD_AUTHENT:

irqEn = 0x12;

waitFor = 0x10;

break;

case PCD_TRANSCEIVE:

irqEn = 0x77;

waitFor = 0x30;

break;

default:

break;

}

WriteRawRC(ComIEnReg, irqEn | 0x80);

ClearBitMask(ComIrqReg, 0x80);

WriteRawRC(CommandReg, PCD_IDLE);

SetBitMask(FIFOLevelReg, 0x80);

for (i = 0; i < InLenByte; i++) { WriteRawRC(FIFODataReg, pInData[i]); }

WriteRawRC(CommandReg, Command);

if (Command == PCD_TRANSCEIVE) { SetBitMask(BitFramingReg, 0x80); }

i = 1000;

do {

n = ReadRawRC(ComIrqReg);

i--;

} while ((i != 0) && !(n & 0x01) && !(n & waitFor));

ClearBitMask(BitFramingReg, 0x80);

if (i != 0) {

if (!(ReadRawRC(ErrorReg) & 0x1B)) {

status = MI_OK;

if (n & irqEn & 0x01) { status = MI_NOTAGERR; }

if (Command == PCD_TRANSCEIVE) {

n = ReadRawRC(FIFOLevelReg);

lastBits = ReadRawRC(ControlReg) & 0x07;

if (lastBits) { *pOutLenBit = (n - 1) * 8 + lastBits; }

else { *pOutLenBit = n * 8; }

if (n == 0) { n = 1; }

if (n > MAXRLEN) { n = MAXRLEN; }

for (i = 0; i < n; i++) { pOutData[i] = ReadRawRC(FIFODataReg); }

}

} else { status = MI_ERR; }

}

SetBitMask(ControlReg, 0x80);

WriteRawRC(CommandReg, PCD_IDLE);

return status;

}

char PcdRequest(unsigned char req_code, unsigned char *pTagType) {

int status;

unsigned int unLen;

unsigned char ucComMF522Buf[MAXRLEN];

ClearBitMask(Status2Reg, 0x08);

WriteRawRC(BitFramingReg, 0x07);

SetBitMask(TxControlReg, 0x03);

ucComMF522Buf[0] = req_code;

status = PcdComMF522(PCD_TRANSCEIVE, ucComMF522Buf, 1, ucComMF522Buf, &unLen);

if ((status == MI_OK) && (unLen == 0x10)) {

*pTagType = ucComMF522Buf[0];

*(pTagType + 1) = ucComMF522Buf[1];

} else {

status = MI_ERR;

}

return status;

}

char PcdAnticoll(unsigned char *pSnr) {

int status;

unsigned char i, snr_check = 0;

unsigned int unLen;

unsigned char ucComMF522Buf[MAXRLEN];

ClearBitMask(Status2Reg, 0x08);

WriteRawRC(BitFramingReg, 0x00);

ClearBitMask(CollReg, 0x80);

ucComMF522Buf[0] = PICC_ANTICOLL1;

ucComMF522Buf[1] = 0x20;

status = PcdComMF522(PCD_TRANSCEIVE, ucComMF522Buf, 2, ucComMF522Buf, &unLen);

if (status == MI_OK) {

for (i = 0; i < 4; i++) {

*(pSnr + i) = ucComMF522Buf[i];

snr_check ^= ucComMF522Buf[i];

}

if (snr_check != ucComMF522Buf[i]) { status = MI_ERR; }

}

SetBitMask(CollReg, 0x80);

return status;

}

char PcdSelect(unsigned char *pSnr) {

int status;

unsigned char i;

unsigned int unLen;

unsigned char ucComMF522Buf[MAXRLEN];

ucComMF522Buf[0] = PICC_ANTICOLL1;

ucComMF522Buf[1] = 0x70;

ucComMF522Buf[6] = 0;

for (i = 0; i < 4; i++) {

ucComMF522Buf[i + 2] = *(pSnr + i);

ucComMF522Buf[6] ^= *(pSnr + i);

}

CalulateCRC(ucComMF522Buf, 7, &ucComMF522Buf[7]);

ClearBitMask(Status2Reg, 0x08);

status = PcdComMF522(PCD_TRANSCEIVE, ucComMF522Buf, 9, ucComMF522Buf, &unLen);

if ((status == MI_OK) && (unLen == 0x18)) { status = MI_OK; }

else { status = MI_ERR; }

return status;

}

char PcdRead(unsigned char addr, unsigned char *pData) {

int status;

unsigned int unLen;

unsigned char i, ucComMF522Buf[MAXRLEN];

ucComMF522Buf[0] = PICC_READ;

ucComMF522Buf[1] = addr;

CalulateCRC(ucComMF522Buf, 2, &ucComMF522Buf[2]);

status = PcdComMF522(PCD_TRANSCEIVE, ucComMF522Buf, 4, ucComMF522Buf, &unLen);

if ((status == MI_OK) && (unLen == 0x90))

{

for (i = 0; i < 16; i++) { *(pData + i) = ucComMF522Buf[i]; }

} else { status = MI_ERR; }

return status;

}

char PcdWrite(unsigned char addr, unsigned char *pData) {

int status;

unsigned int unLen;

unsigned char i, ucComMF522Buf[MAXRLEN];

ucComMF522Buf[0] = PICC_WRITE;

ucComMF522Buf[1] = addr;

CalulateCRC(ucComMF522Buf, 2, &ucComMF522Buf[2]);

status = PcdComMF522(PCD_TRANSCEIVE, ucComMF522Buf, 4, ucComMF522Buf, &unLen);

if ((status != MI_OK) || (unLen != 4) || ((ucComMF522Buf[0] & 0x0F) != 0x0A)) { status = MI_ERR; }

if (status == MI_OK) {

for (i = 0; i < 16; i++) { ucComMF522Buf[i] = *(pData + i); }

CalulateCRC(ucComMF522Buf, 16, &ucComMF522Buf[16]);

status = PcdComMF522(PCD_TRANSCEIVE, ucComMF522Buf, 18, ucComMF522Buf, &unLen);

if ((status != MI_OK) || (unLen != 4) || ((ucComMF522Buf[0] & 0x0F) != 0x0A)) { status = MI_ERR; }

}

return status;

}

char PcdHalt(void) {

int status;

unsigned int unLen;

unsigned char ucComMF522Buf[MAXRLEN];

ucComMF522Buf[0] = PICC_HALT;

ucComMF522Buf[1] = 0;

CalulateCRC(ucComMF522Buf, 2, &ucComMF522Buf[2]);

status = PcdComMF522(PCD_TRANSCEIVE, ucComMF522Buf, 4, ucComMF522Buf, &unLen);

return status;

}

unsigned char cardLeave(void) {

unsigned char card_status = 0;

unsigned char status;

static unsigned char put_card_flag = 0;

static unsigned char take_card_flag = 0;

unsigned char buf[20];

status = PcdRequest(PICC_REQALL, buf);

if (status != MI_OK) {

card_status = 2;

}

status = PcdAnticoll(buf);

if (status != MI_OK) {

card_status = 2;

} else card_status = 1;

PcdHalt();

switch (card_status) {

case 1:

put_card_flag = 1;

return 0;

case 2:

take_card_flag = 1;

if ((1 == put_card_flag) && (1 == take_card_flag)) {

put_card_flag = 0;

take_card_flag = 0;

return 1;

}

break;

}

return 0;

}

unsigned char *readCard(unsigned char *length) {

unsigned char status, card_leave;

static unsigned char strp[20];

static unsigned char strb[20];

static unsigned char cardidbuf[20];

*length = 0;

char *p = NULL;

card_leave = cardLeave();

if (1 == card_leave) memset(cardidbuf, 0, 20);

sprintf(strb, "%x %x %x %x", cardidbuf[0], cardidbuf[1], cardidbuf[2], cardidbuf[3]);

status = PcdRequest(PICC_REQALL, cardidbuf);

if (status != MI_OK) {

return p;

}

status = PcdAnticoll(cardidbuf);

if (status != MI_OK) {

return p;

}

PcdHalt();

sprintf(strp, "%x %x %x %x", cardidbuf[0], cardidbuf[1], cardidbuf[2], cardidbuf[3]);

*length = 4;

if (strcmp(strb, strp) == 0) return p;

else return strp;

}

void rc523Init(void) {

PcdReset();

Delay_ms(250);

PcdAntennaOff();

PcdAntennaOn();

}

- 在card.h中加入芯片寄存器定义及API声明：

#ifndef CODE_RC523_H

#define CODE_RC523_H

// PAGE 0

#define     RFU00                 0x00

#define     CommandReg            0x01

#define     ComIEnReg             0x02

#define     DivlEnReg             0x03

#define     ComIrqReg             0x04

#define     DivIrqReg             0x05

#define     ErrorReg              0x06

#define     Status1Reg            0x07

#define     Status2Reg            0x08

#define     FIFODataReg           0x09

#define     FIFOLevelReg          0x0A

#define     WaterLevelReg         0x0B

#define     ControlReg            0x0C

#define     BitFramingReg         0x0D

#define     CollReg               0x0E

#define     RFU0F                 0x0F

// PAGE 1

#define     RFU10                 0x10

#define     ModeReg               0x11

#define     TxModeReg             0x12

#define     RxModeReg             0x13

#define     TxControlReg          0x14

#define     TxAutoReg             0x15

#define     TxSelReg              0x16

#define     RxSelReg              0x17

#define     RxThresholdReg        0x18

#define     DemodReg              0x19

#define     RFU1A                 0x1A

#define     RFU1B                 0x1B

#define     MifareReg             0x1C

#define     RFU1D                 0x1D

#define     RFU1E                 0x1E

#define     SerialSpeedReg        0x1F

// PAGE 2

#define     RFU20                 0x20

#define     CRCResultRegM         0x21

#define     CRCResultRegL         0x22

#define     RFU23                 0x23

#define     ModWidthReg           0x24

#define     RFU25                 0x25

#define     RFCfgReg              0x26

#define     GsNReg                0x27

#define     CWGsCfgReg            0x28

#define     ModGsCfgReg           0x29

#define     TModeReg              0x2A

#define     TPrescalerReg         0x2B

#define     TReloadRegH           0x2C

#define     TReloadRegL           0x2D

#define     TCounterValueRegH     0x2E

#define     TCounterValueRegL     0x2F

// PAGE 3

#define     RFU30                 0x30

#define     TestSel1Reg           0x31

#define     TestSel2Reg           0x32

#define     TestPinEnReg          0x33

#define     TestPinValueReg       0x34

#define     TestBusReg            0x35

#define     AutoTestReg           0x36

#define     VersionReg            0x37

#define     AnalogTestReg         0x38

#define     TestDAC1Reg           0x39

#define     TestDAC2Reg           0x3A

#define     TestADCReg            0x3B

#define     RFU3C                 0x3C

#define     RFU3D                 0x3D

#define     RFU3E                 0x3E

#define     RFU3F		  0x3F

#define PCD_IDLE              0x00               //取消当前命令

#define PCD_AUTHENT           0x0E               //验证密钥

#define PCD_RECEIVE           0x08               //接收数据

#define PCD_TRANSMIT          0x04               //发送数据

#define PCD_TRANSCEIVE        0x0C               //发送并接收数据

#define PCD_RESETPHASE        0x0F               //复位

#define PCD_CALCCRC           0x03               //CRC计算

#define PICC_REQIDL           0x26               //寻天线区内未进入休眠状态

#define PICC_REQALL           0x52               //寻天线区内全部卡

#define PICC_ANTICOLL1        0x93               //防冲撞

#define PICC_ANTICOLL2        0x95               //防冲撞

#define PICC_AUTHENT1A        0x60               //验证A密钥

#define PICC_AUTHENT1B        0x61               //验证B密钥

#define PICC_READ             0x30               //读块

#define PICC_WRITE            0xA0               //写块

#define PICC_DECREMENT        0xC0               //扣款

#define PICC_INCREMENT        0xC1               //充值

#define PICC_RESTORE          0xC2               //调块数据到缓冲区

#define PICC_TRANSFER         0xB0               //保存缓冲区中数据

#define PICC_HALT             0x50               //休眠

#define MI_OK                          0

#define MI_NOTAGERR                    (-1)

#define MI_ERR                         (-2)

#define DEF_FIFO_LENGTH       64                 //FIFO size=64byte

#define RC523_PORT GPIOF

void rc523Init(void);

unsigned char* readCard(unsigned char *length);

#endif

- 修改board.c，在boardInit函数中增加读卡模块初始化代码。

#include "stm8l15x.h"

#include "led.h"

#include "timer4.h"

#include "oled.h"

#include "tsm12.h"

#include "card.h"

void boardInit(void){

…

//card

GPIO_Init(GPIOF,GPIO_Pin_5|GPIO_Pin_6|GPIO_Pin_7,

GPIO_Mode_Out_PP_High_Fast);

GPIO_Init(GPIOF, GPIO_Pin_4, GPIO_Mode_In_PU_No_IT);

rc523Init();

}

- 修改main函数，在main函数中循环读卡，读到卡号后，把卡号显示到屏幕上。

#include <stdio.h>

#include "stm8l15x.h"

#include "stm8_eval.h"

#include "board.h"

#include "led.h"

#include "door.h"

#include "sound.h"

#include "oled.h"

#include "tsm12.h"

#include "card.h"

void main(void) {

unsigned char length;

unsigned char* cardId;

CLK_SYSCLKDivConfig(CLK_SYSCLKDiv_1);

boardInit();

display_GB2312_string(0,0,"智能锁实践", 0);

while (1) {

/*请在此调用readCard 函数，读到卡号时把卡号显示到屏幕上，并执行一次开锁*/

/*注意，当没有卡片时，readCard 函数的返回值是NULL*/

}

}

- 编译并烧写程序到实验平台。

5  实验结果

重启单片机，把IC卡放到读卡模块上，单片机将读到该卡的卡号并显示在OLED屏幕上。