实验九、SIM7020模块驱动实验

1  实验目的

设计SIM7020模块驱动，实现NB无线通信功能。

2  实验内容

1、打开“指静脉识别模块驱动”工程。

2、添加SIM7020模块到工程中。

3、编写代码，在board中实现SIM7020模块初始化。

4、编写代码，通过SIM7020模块实现连接中国移动ONENET服务器，并使用该服务器实现远程开锁功能。

5、编译工程，验证SIM7020模块驱动是否完成。

3  实验原理

NB-IOT是窄带物联网（Narrow BandInternetof Things）的简写，具有以下特点：

- 强链接，比现有技术提高50-100倍的接入数量，满足大量设备联网需求。

- 高覆盖，比LTE提升 20dBm 增益，适合地下车库，深井等应用场景。

- 低功耗，超低的功耗可以将电池供电使用时间提升到数年之久。

- 低成本，复用现有基站设备，相比LoRa无需重新建网，节省成本。

SIM7020是一款多频段NB-IoT无线通信模块，模块采用42 PIN LCC封装。SIM7020拥有丰富的硬件接口，包括串口，GPIO，ADC等，这也使得模块具备丰富的扩展性，为用户的产品开发提供了极大的便利性。SIM7020的封装和SIM800C模块兼容，可以尽可能缩短客户研发时间，加快客户产品投放市场的速度。SIM7020模块是低延迟、低功耗、低吞吐量应用的最优解决方案，非常适用于如表计、远程控制、资产跟踪、远程监控、远程医疗、共享单车等物联网应用。

使用SIM7020模块的通信结构图如下图所示，数据发送终端使用串口与SIM7020模块通信，SIM7020模块通过NB-IOT基站链接云平台，并作为数据传输中介，把终端与服务器之间的数据进行相互转发，实现终端联网功能。

SIM7020与主板连接电路图如下图所示。

（1）SIM7020 AT命令

SIM7020采用AT命令的方式与单片机通信，该模块的命令非常多，为了简化实验，本实验中只用了关键的几个命令，更详细命令请参考《9.SIM7020 Series_AT Command Manual_V1.01.pdf》。

| 指令 | 功能描述 |
| --- | --- |
| AT+CPSMS | 设置节电模式 |
| AT+CMQNEW | 创建TCP连接 |
| AT+CMQCON | 建立MQTT连接 |
| AT+CMQSUB | 订阅主题 |
| AT+CMQPUB | 发布主题 |
| +CMQPUB | 新消息到达提示 |
| AT+CMQDISCON | 断开连接 |

1）AT+CPSMS

|  | 说明 | 示例 |
| --- | --- | --- |
| 功能 | 设置节电模式 |  |
| 查询 | 查询当前工作模式 | AT+ CPSMS? |
| 设置 | 设置当前工作模式 | AT+CPSMS=0 |

2）AT+CMQNEW

|  | 说明 | 示例 |
| --- | --- | --- |
| 功能 | 创建TCP连接 | / |
| 查询 | 查询当前已建立的TCP连接 | AT+CMQNEW? |
| 设置 | 建立TCP连接 | AT+CMQNEW="183.230.40.39","6002",12000,100 |

3）AT+ CMQCON

|  | 说明 | 示例 |
| --- | --- | --- |
| 功能 | 建立MQTT连接 | / |
| 查询 | 查询当前已建立的MQTT连接 | AT+ CMQCON? |
| 设置 | 建立MQTT连接 | AT+CMQCON=0,4,"690504327",180,0,0,407222,12345678 |

4）AT+CMQSUB

|  | 说明 | 示例 |
| --- | --- | --- |
| 功能 | 订阅主题 | AT+CMQSUB=0,"test",0 |

5）AT+CMQPUB

|  | 说明 | 示例 |
| --- | --- | --- |
| 功能 | 发布主题 | AT+CMQPUB=0,"$dp",0,0,0,30,03000C7B2274657374223A3132337D |

6）+CMQPUB

|  | 说明 | 示例 |
| --- | --- | --- |
| 功能 | 新消息到达提示 | +CMQPUB: 0,"$creq/b931ed81-b089-552a-b1bf-d8f0be60d5bd",0,0,0,16,"3132333435363738" |

7）AT+CMQDISCON

|  | 说明 | 示例 |
| --- | --- | --- |
| 功能 | 断开连接 | AT+CMQDISCON=0 |

（2）在ONENET中添加设备

注册并登陆中国移动ONENET平台：https://open.iot.10086.cn/，进入控制台：

点击“多协议接入”：

点击添加产品：

填写产品信息，然后点击“确定”：

添加设备完成，记下产品ID：

点击“设备列表”->“添加设备”：

填写设备信息，然后点击“添加”：

添加设备完成：

在上图点击“详情”，记下“设备ID”和“鉴权信息”：

（3）上线测试

在信号切换菜单中切换调试串口直连SIM7020模块。

使用串口调试助手打开实验平台的调试串口，波特率选择“9600”，注意要勾上“加回车换行”：

SIM7020进入PSM模式后将不会响应AT命令，所以测试开始前先关闭PSM功能：

1）连接ONENET服务器

发送命令“AT+CMQNEW="183.230.40.39","6002",12000,100”。

模块返回“OK”。

2）建立MQTT连接

发送命令“AT+CMQCON=0,4,"739126600",180,0,0,407222,123456789”，其中739126600、407222和123456789分别为设备ID、产品ID和鉴权信息。

模块返回“OK”

3）订阅主题

发送命令“AT+CMQSUB=0,"test",0”，其中“test”为要订阅的主题。

模块返回“OK”。

4）上报数据点

发送命令“AT+CMQPUB=0,"$dp",0,0,0,30,03000C7B2274657374223A3132337D”。

模块返回“OK”。

其中“03000C7B2274657374223A3132337D”为《9.设备终端接入协议-MQTT.docx》中规定的数据点上报格式，以十六进制字符串来表示。“03”：JSON格式字符串；“000C”：JSON字符串长度为12字节；“7B2274657374223A3132337D”：JSON字符串“{"test":123}”

数据格式定义如下：

| Byte 1 | 数据点类型指示：type=3 // JSON格式字符串 |
| --- | --- |
| Byte 2 | //指示后面字符串长度 固定两字节长度高位字节 |
| Byte 3 | 固定两字节长度低位字节 |
| Byte 4 | 通用格式： { “datastream_id1”:”value1”, “datastream_id2”:”value2”, … } 示例： {“temperature”:22.5,”humidity”:”95.2%”} |
| … … … … |  |
| Byte n |  |

可以在ONENET平台上查看上报的数据点，点击设备列表中的“数据流”：

可以看到上报了名为“test”，值为“123”的数据点：

5）平台下发命令

在ONENET平台上点击“下发命令”：

再点击“下发命令”：

点击“字符串”，输入数值“123”，再点击发送：

在串口调试助手中收到消息如下，其中“313233”为收到的十六进制表示的“123”：

6）断开连接

通信结束后发送“AT+CMQDISCON=0”断开连接。

模块返回“OK”。

（3）软件设计

设计SIM7020模块状态图如上图所示。系统启动后，状态为“初始化”；初始化完成后，切换到“关闭PSM”；然后开始TPC连接和MQTT连接，连接成功后订阅主题，然后开始等待消息，此时也可以发布主题。

4  实验步骤

- 使用Clion切换代码到“指静脉识别模块驱动”版本。

- 启动IAR集成开发环境，选择【File】->【Open Workspace】，打开该工程，如下图：

- 在driver目录下添加sim7020.c和sim7020.h，如下图所示：

- 右键【driver】->【Add】->【Add Files..】，添加sim7020.c和sim7020.h到工程中如下图所示：

- 在sim7020.c中加入SIM7020模块驱动代码。其中需要替换DEVICE_ID、PRODUCT_ID和AUTH_INFO需要替换为自己新建的设备信息。

#include "stdio.h"

#include "stm8l15x.h"

#include <string.h>

#include "usart.h"

#include "timer4.h"

#include "SIM7020.h"

#include "door.h"

#include "sound.h"

#include "led.h"

typedef enum {

NB_INIT,

NB_DISABLE_PSM,

NB_DISCONNECT,

NB_WAIT_TCP_CONNECTION,

NB_WAIT_MQTT_CONNECTION,

NB_SUBSCRIBE_TOPIC_DELAY,

NB_SUBSCRIBE_TOPIC,

NB_WAIT_MESSAGE,

} SIM7020_Status;

#define SERVER_ADDRESS "\"183.230.40.39\""

#define SERVER_PORT "\"6002\""

#define DEVICE_ID "\"690504327\""

#define PRODUCT_ID "407222"

#define AUTH_INFO "12345678"

#define SUBSCRIBE_TOPIC "\"test\""

#define PUB_TOPIC "\"$dp\""

#define NB_HEART_BEAT_PERIOD 10000

#define MAX_NB_WAIT_TIME 10000

#define NB_RETRY_WAIT_TIME 3000

#define NB_RECEIVE_BUF_LEN 120

static unsigned char receiveBuf[NB_RECEIVE_BUF_LEN];

static unsigned char receiveBufLength = 0;

static unsigned char status = NB_INIT;

static u32 startTime = 0;

void onSIM7020ReceiveByte(unsigned char byte) {

if (receiveBufLength < NB_RECEIVE_BUF_LEN) {

receiveBuf[receiveBufLength] = byte;

receiveBufLength++;

}

}

void createTcpConnection(void) {

//AT+CMQNEW="183.230.40.39","6002",12000,100

sendBytes(USART3, "AT+CMQNEW=", 10);

sendBytes(USART3, SERVER_ADDRESS, sizeof(SERVER_ADDRESS));

sendBytes(USART3, ",", 1);

sendBytes(USART3, SERVER_PORT, sizeof(SERVER_PORT));

sendBytes(USART3, ",12000,100\r\n", 12);

}

void createMQTTConnection() {

//    AT+CMQCON=0,4,"690504327",180,0,0,407222,12345678

sendBytes(USART3, "AT+CMQCON=0,4,", 14);

sendBytes(USART3, DEVICE_ID, sizeof(DEVICE_ID));

sendBytes(USART3, ",180,0,0,", 9);

sendBytes(USART3, PRODUCT_ID, sizeof(PRODUCT_ID));

sendBytes(USART3, ",", 1);

sendBytes(USART3, AUTH_INFO, sizeof(AUTH_INFO));

sendBytes(USART3, "\r\n", 2);

}

void subscribeTopic() {

//    AT+CMQSUB=0,"test",0   AT+CMQSUB=0,"test",0

sendBytes(USART3, "AT+CMQSUB=0,", 12);

sendBytes(USART3, SUBSCRIBE_TOPIC, sizeof(SUBSCRIBE_TOPIC));

sendBytes(USART3, ",0\r\n", 4);

}

static unsigned char checkIfReceivePackage(void) {

if (receiveBufLength > 2

&& receiveBuf[receiveBufLength - 2] == '\r'

&& receiveBuf[receiveBufLength - 1] == '\n') {

return 1;

}

return 0;

}

static void clearBuf(void) {

receiveBufLength = 0;

memset(receiveBuf, 0, NB_RECEIVE_BUF_LEN);

}

static void bytesToHexString(unsigned char *inBuf, unsigned char inLength, unsigned char *outBuf) {

char i, hexArray[] = "0123456789ABCDEF";

for (i = 0; i < inLength; i++) {

outBuf[i * 2] = hexArray[inBuf[i] >> 4];

outBuf[i * 2 + 1] = hexArray[inBuf[i] & 0x0f];

}

outBuf[i * 2] = '\0';

}

//AT+CMQPUB=0,"$dp",0,0,0,30,03000C7B2274657374223A3132337D

//                                 7B2274657374223A3132337D = {"test":123}

void sendDataBySIM7020(unsigned char *pData, unsigned char length) {

char sendBuf[40], printBuf[20];

unsigned char jsonLength = 0;

sendBytes(USART3, "AT+CMQPUB=0,", 12);

sendBytes(USART3, PUB_TOPIC, strlen(PUB_TOPIC));

sendBytes(USART3, ",0,0,0,", 7);

jsonLength = 9 + length;

length = jsonLength + 3;

sprintf(sendBuf, "%d", length * 2);

sendBytes(USART3, (unsigned char *) sendBuf, strlen((char const *) sendBuf));

sendBytes(USART3, ",0300", 5);

bytesToHexString(&jsonLength, 1, (unsigned char *) sendBuf);

sendBytes(USART3, (unsigned char *) sendBuf, strlen(sendBuf));

sprintf(printBuf, "{\"test\":");

bytesToHexString((unsigned char *) printBuf, 8, (unsigned char *) sendBuf);

sendBytes(USART3, (unsigned char *) sendBuf, strlen(sendBuf));

sprintf(printBuf, "%s}", pData);

bytesToHexString((unsigned char *) printBuf, strlen(printBuf), (unsigned char *) sendBuf);

sendBytes(USART3, (unsigned char *) sendBuf, strlen(sendBuf));

sendBytes(USART3, "\r\n", 2);

}

//+CMQPUB: 0,"$creq/b931ed81-b089-552a-b1bf-d8f0be60d5bd",0,0,0,16,"3132333435363738"

static void parseNBData(unsigned char *pNBData, unsigned char *outBuf, unsigned char *outLength) {

unsigned char index = 0, i = 0, j = 0, byte[2];

*outLength = 0;

//找到第5个逗号

while (j < 5) {

if (pNBData[index] == ',') {

j++;

}

if (++index > receiveBufLength) {

return;

}

}

//读取数据长度

while (pNBData[index] != ',') {

*outLength = *outLength * 10 + pNBData[index] - '0';

if (++index > receiveBufLength) {

*outLength = 0;

return;

}

}

*outLength /= 2;

index += 2;

//读取数据

for (i = 0; i < *outLength; i++) {

for (j = 0; j < 2; j++) {

if (pNBData[index + j] >= '0' && pNBData[index + j] <= '9') {

byte[j] = pNBData[index + j] - '0';

} else if (pNBData[index + j] >= 'A' && pNBData[index + j] <= 'F') {

byte[j] = pNBData[index + j] - 'A' + 10;

} else if (pNBData[index + j] >= 'a' && pNBData[index + j] <= 'f') {

byte[j] = pNBData[index + j] - 'a' + 10;

}

}

outBuf[i] = byte[0] << 4 | byte[1];

index += 2;

if (index > receiveBufLength) {

*outLength = 0;

return;

}

}

}

static void parseResult(void) {

unsigned char dataLength = 0;

switch (status) {

case NB_DISABLE_PSM:

if (strstr((char const *) receiveBuf, "OK") != 0 ||

strstr((char const *) receiveBuf, "ERROR") != 0) {

clearBuf();

sendBytes(USART3, "AT+CMQDISCON=0\r\n", 16);//断开之前的连接

status = NB_DISCONNECT;

startTime = Timer_GetTickCount();

printf("disconnect\n");

}

break;

case NB_DISCONNECT:

if (strstr((char const *) receiveBuf, "OK") != 0 ||

strstr((char const *) receiveBuf, "ERROR") != 0) {

clearBuf();

createTcpConnection();//开始连接服务器

status = NB_WAIT_TCP_CONNECTION;

startTime = Timer_GetTickCount();

printf("tcp connect\n");

}

break;

case NB_WAIT_TCP_CONNECTION:

if (strstr((char const *) receiveBuf, "OK") != 0) {

clearBuf();

createMQTTConnection();//创建MQTT连接

status = NB_WAIT_MQTT_CONNECTION;

startTime = Timer_GetTickCount();

printf("mqtt connect\n");

}

break;

case NB_WAIT_MQTT_CONNECTION:

if (strstr((char const *) receiveBuf, "OK") != 0) {

clearBuf();

status = NB_SUBSCRIBE_TOPIC_DELAY;

startTime = Timer_GetTickCount();

}

break;

case NB_SUBSCRIBE_TOPIC:

if (strstr((char const *) receiveBuf, "OK") != 0) {

clearBuf();

status = NB_WAIT_MESSAGE;

startTime = Timer_GetTickCount();

printf("wait message\n");

}

break;

case NB_WAIT_MESSAGE:

if (strstr((char const *) receiveBuf, "$creq") != 0) {

parseNBData(receiveBuf, receiveBuf, &dataLength);

if (dataLength > 0) {

unlockDoor();

playSound(31);

flashLedByIndex(12);

}

}

clearBuf();

break;

}

}

void SIM7020Process(void) {

switch (status) {

case NB_INIT:

status = NB_DISABLE_PSM;

startTime = Timer_GetTickCount();

clearBuf();

sendBytes(USART3, "AT+CPSMS=0\r\n", 12);

break;

case NB_DISABLE_PSM:

case NB_DISCONNECT:

case NB_WAIT_TCP_CONNECTION:

case NB_WAIT_MQTT_CONNECTION:

case NB_SUBSCRIBE_TOPIC_DELAY:

case NB_SUBSCRIBE_TOPIC:

if (Timer_PassedDelay(startTime, NB_RETRY_WAIT_TIME)) {

if (status == NB_SUBSCRIBE_TOPIC_DELAY) {//等待时间到

subscribeTopic();//订阅主题

status = NB_SUBSCRIBE_TOPIC;

startTime = Timer_GetTickCount();

printf("subscribe\n");

} else {

status = NB_INIT;

printf("time out:[%s],%d\n", receiveBuf, receiveBufLength);

break;

}

}

case NB_WAIT_MESSAGE:

if(Timer_PassedDelay(startTime, NB_HEART_BEAT_PERIOD)){

sendDataBySIM7020("0",1);

startTime = Timer_GetTickCount();

}

if (!checkIfReceivePackage()) break;

printf("recv:%s\n", receiveBuf);

parseResult();

break;

}

}

- 在sim7020.h中加入api声明：

#ifndef CODE_SIM7020_H

#define CODE_SIM7020_H

void onSIM7020ReceiveByte(unsigned char byte);

void sendDataBySIM7020(unsigned char *pData, unsigned char length);

void SIM7020Process(void);

#endif

- 修改board.c，在boardInit函数中加入USART3初始化：

#include "stm8l15x.h"

#include "led.h"

#include "timer4.h"

#include "oled.h"

#include "tsm12.h"

#include "card.h"

#include "usart.h"

#include "fingerprint.h"

void boardInit(void){

…

//SIM7020 & ESP32

USART3_Config();

}

- 修改stm8l15x_it.c，在INTERRUPT_HANDLER(TIM3_CC_USART3_RX_IRQHandler, 22)函数中调用onNB73ReceiveByte:

#include "sim7020.h"

INTERRUPT_HANDLER(TIM3_CC_USART3_RX_IRQHandler, 22)

{

if(USART_GetITStatus(USART3,USART_IT_RXNE) != RESET)

{

unsigned char rsData = USART_ReceiveData8(USART3);

onSIM7020ReceiveByte(rsData);

USART_ClearITPendingBit(USART3,USART_IT_RXNE);

}

}

- 修改main.c，调用SIM7020Process函数。

#include <stdio.h>

#include "stm8l15x.h"

#include "stm8_eval.h"

#include "board.h"

#include "led.h"

#include "door.h"

#include "sound.h"

#include "oled.h"

#include "tsm12.h"

#include "card.h"

#include "fingerprint.h"

#include "fingervein.h"

#include "sim7020.h"

void main(void) {

CLK_SYSCLKDivConfig(CLK_SYSCLKDiv_1);

boardInit();

display_GB2312_string(0,0,"智能锁实践", 0);

while (1) {

/*请在此调用SIM7020Process函数*/

}

}

- 编译并烧写程序到实验平台。

5  实验结果

在信号切换菜单中切换主控制器直连SIM7020模块：

重启设备后，可以在ONENET平台中查询到设备已经上线：

此时在ONENET平台上下发任意数字给设备，设备将执行一次开锁动作。